Computer Networking Notes

Beginner to Advanced (Cybersecurity Focus)

# 1. How Networking Was Born   [before 5:43]

From military experiments to the internet we use every day.

| Decade | What happened |
| --- | --- |
| 1960s | ARPANET created by the US Defense Department. First network, connecting 4 universities. |
| 1970s | TCP/IP protocols invented. Email introduced for the first time. |
| 1980s | DNS system created. Internet opens up to more institutions. |
| 1990s | World Wide Web invented by Tim Berners-Lee. Internet goes public and commercial. |
| 2000s | Wi-Fi becomes mainstream. Broadband replaces dial-up globally. |
| 2010s | Cloud computing explodes. Mobile internet dominates traffic. |
| 2020s | 5G rolls out globally. IoT connects billions of devices. |

# Fun facts from the slide

- 1969 The first message ever sent over ARPANET was "LO". The system crashed after 2 letters.

- TCP/IP The protocol suite invented in 1974 still powers the internet today.

- Hacking The first computer hack was recorded in 1971, on phone networks.

- Instructor emphasis Network = connect (computers connecting to each other).

# 2. What is Networking & Why Hackers Must Understand It   [5:43]

Definition: Networking enables computers to communicate and share resources through interconnected systems, using standardized protocols and addressing schemes.

Why it matters for ethical hackers:

| Skill area | What it means |
| --- | --- |
| Packet Sniffing & Analysis | Intercept and decode network traffic to find vulnerabilities and sensitive data being transmitted. |
| Protocol Exploitation | Understand weaknesses in the TCP/IP stack to carry out attacks such as session hijacking or DNS poisoning. |
| Network Reconnaissance | Map target infrastructure with tools like Nmap to discover hosts, open ports and running services. |
| Firewall Evasion | Bypass security controls by manipulating packet headers and understanding filtering mechanisms. |

# 3. Types of Networks   [12:28]

| Type | Full form | Description |
| --- | --- | --- |
| LAN | Local Area Network | Connects devices within a limited geographical area, such as an office building or campus. |
| MAN | Metropolitan Area Network | Covers a city or large town. Larger than a LAN but smaller than a WAN. |
| WAN | Wide Area Network | Spans large distances across cities, countries or continents, connecting multiple LANs. |
| PAN | Personal Area Network | Connects devices within a personal workspace, such as smartphone, laptop and peripherals. |

# Fiber Optic Cables (Undersea Cables)   [16:33]

- WAN Wide Area Network connects countries. Instructor note: "countries".

- The slide shows a diver working on an undersea cable and a submarine-cable map around India, the Arabian Sea, the Bay of Bengal, Sri Lanka, the Maldives and Southeast Asia.

- Takeaway Cables laid on the sea floor are what physically link countries and continents, so most international internet traffic travels under the ocean.

# 4. Network Devices   [18:13]

| Device | Function |
| --- | --- |
| Router | Forwards packets between different networks using IP addresses. |
| Switch | Connects devices on the same network using MAC addresses. |
| Hub | Broadcasts data to all connected devices (obsolete technology). |
| Firewall | Filters traffic based on security rules, blocking unauthorized access. |
| Modem | Converts digital signals to analogue for internet connectivity. |
| Access Point | Provides wireless network connectivity to devices. |

Instructor marked: Router, Switch and Hub (tick marks on these three).

# 5. IP Address vs MAC Address   [24:47]

|  | IP Address | MAC Address |
| --- | --- | --- |
| Definition | Logical address assigned by the network. | Physical address burned into the hardware. |
| Changes? | Changes when the device moves networks. | Permanent and unique to the device. Never changes. |
| Assigned by | Network administrator or DHCP. | Manufacturer (built into the hardware). |
| Used for | Routing across networks. | Local network communication. |

Instructor sketch: a device uses its IP address to reach the Internet.

# 6. IPv4 vs IPv6 Comparison   [28:08]

|  | IPv4 | IPv6 |
| --- | --- | --- |
| Format | 32-bit, dotted decimal notation | 128-bit, hexadecimal notation |
| Example | 192.168.1.1 | 2001:0db8::1 |
| Structure | 4 octets (8 bits each): 192 . 168 . 1 . 1 | 8 groups (16 bits each): 2001 : 0db8 : 0000 : 0000 : 0000 : 0000 : 0000 : 0001 |

Instructor example: 192.168.10.1 is a 32-bit address.

# 7. Public vs Private IP Addresses   [36:26]

|  | Public IP | Private IP |
| --- | --- | --- |
| Definition | Globally unique address, routable on the internet. | Non-routable address used within private networks. |
| Example | 8.8.8.8 (Google DNS) | 192.168.1.100 |
| Use case | Servers, websites, devices accessible from the internet. | Home networks, corporate LANs, internal devices. |

Private IP ranges:

| Class | Range |
| --- | --- |
| Class A | 10.0.0.0 to 10.255.255.255 |
| Class B | 172.16.0.0 to 172.31.255.255 |
| Class C | 192.168.0.0 to 192.168.255.255 |

Important: Private IPs are not routable on the internet.

# 8. Static vs Dynamic IP Assignment   [39:52]

|  | Static IP | Dynamic IP |
| --- | --- | --- |
| Definition | Manually assigned, fixed address. | Automatically assigned by a DHCP server. |
| Behaviour | Does not change. | Changes periodically or when reconnecting. |
| Assigned by | Administrator. | DHCP protocol. |
| Used for | Servers, printers, network devices. | Home networks, mobile devices. |

DHCP flow:

- Device connects to the network.

- DHCP assigns an IP address.

- Device uses that IP.

- Lease expires, so the device renews the same IP or gets a new one.

# 9. IP Address Classes (Classful Addressing)   [42:46 and 49:15]

Definition: The original method of dividing IPv4 addresses into classes based on the first octet.

| Class | First octet | Network bits | Host bits | Example |
| --- | --- | --- | --- | --- |
| A | 1 to 126 | 8 bits | 24 bits | 10.0.0.0 (16 million hosts) |
| B | 128 to 191 | 16 bits | 16 bits | 172.16.0.0 (65,000 hosts) |
| C | 192 to 223 | 24 bits | 8 bits | 192.168.0.0 (254 hosts) |
| D | 224 to 239 | Reserved | Multicast | 224.0.0.0 (group communication) |
| E | 240 to 255 | Reserved | Experimental | 240.0.0.0 (research only) |

Key points:

- Class A Large networks (few networks, many hosts).

- Class B Medium networks (balanced).

- Class C Small networks (many networks, few hosts).

- Classes D & E Special use, not for standard addressing.

- Instructor underlined Class A, B, C and their host counts: 16 million, 65,000, 254.

Extra note: The slide shows Class A as 1 to 126 and Class B starting at 128. The 127.x.x.x block is skipped because it is reserved for loopback (127.0.0.1, "localhost").

# 10. CIDR Notation   [48:40 and 49:45]

Definition: A way to express an IP address and its network size in one line.

Example: 192.168.1.0/24

- Network portion First 24 bits identify the network (e.g. 192.168.1).

- Host portion Last 8 bits identify the device (.0 to .255).

| CIDR | Subnet mask | Usable hosts |
| --- | --- | --- |
| /8 | 255.0.0.0 | 16,777,214 |
| /16 | 255.255.0.0 | 65,534 |
| /24 | 255.255.255.0 | 254 |
| /26 | 255.255.255.192 | 62 |
| /30 | 255.255.255.252 | 2 |

# 11. What is Subnetting?   [53:33]

Definition: Dividing one large network into smaller, manageable pieces.

Example: one big network 192.168.1.0/24 is split into four /26 subnets, each with 62 usable hosts.

| Subnet | Network | Hosts |
| --- | --- | --- |
| Subnet A | 192.168.1.0/26 | 62 |
| Subnet B | 192.168.1.64/26 | 62 |
| Subnet C | 192.168.1.128/26 | 62 |
| Subnet D | 192.168.1.192/26 | 62 |

# 12. NAT: Network Address Translation   [56:33]

Definition: NAT allows private IP addresses to communicate with the internet by translating them to public IP addresses.

Path of a request:

Private device (192.168.1.100)  →  Router (NAT translation)  →  Public IP (203.0.113.45)  →  Internet  →  Google

Translation step (at the router):

- Private IP is replaced with the public IP for outbound traffic.

- Response traffic is translated back to the private IP for the device.

Why NAT exists:

- IP address conservation Reduces the need for public IPs (billions of devices, limited IPv4 addresses).

- Security Hides the internal network structure from the internet.

- Network flexibility Lets private networks operate independently.

# 13. What is a Protocol and Port?   [1:02:28]

- Protocol Rules and standards for communication between devices. Examples: HTTP, HTTPS, FTP, DNS, SSH, SMTP.

- Port A logical endpoint, or "door", on a device for network communication. Range 0 to 65535 (65,536 ports in total).

Common ports:

| Protocol | Port |
| --- | --- |
| HTTP | 80 |
| HTTPS | 443 |
| FTP | 21 |
| SSH | 22 |
| DNS | 53 |

| Port range | Category |
| --- | --- |
| 0 to 1023 | Well-known ports |
| 1024 to 49151 | Registered ports |
| 49152 to 65535 | Dynamic / private ports |

# 14. OSI Model: All 7 Layers   [1:08:00]

| Layer | Name | Role |
| --- | --- | --- |
| 7 | Application | User interface and network services. |
| 6 | Presentation | Data translation, encryption, compression. |
| 5 | Session | Establishes and manages connections. |
| 4 | Transport | TCP/UDP, error recovery, flow control. |
| 3 | Network | IP addressing and routing packets. |
| 2 | Data Link | MAC addressing and framing. |
| 1 | Physical | Physical transmission of bits. |

Instructor note: OSI has 7 layers.

# 15. TCP/IP Model Explained   [1:14:06]

| Layer | Protocols / role |
| --- | --- |
| Application Layer | HTTP, FTP, DNS, SMTP protocols. |
| Transport Layer | TCP, UDP for reliable data delivery. |
| Internet Layer | IP, ICMP, ARP for addressing and routing. |
| Network Access Layer | Physical hardware and data link protocols. |

# 16. OSI vs TCP/IP Comparison   [1:15:20]

| OSI model | TCP/IP model | Examples |
| --- | --- | --- |
| 7 Application, 6 Presentation, 5 Session | Application | HTTP, DNS, FTP, SMTP |
| 4 Transport | Transport | TCP, UDP |
| 3 Network | Internet | IP, ICMP |
| 2 Data Link, 1 Physical | Network Access | Ethernet, ARP, MAC |

Key idea: The TCP/IP model consolidates OSI's top three layers into a single Application layer and combines the bottom two into the Network Access layer.

# 17. Data Encapsulation & Decapsulation   [1:16:00]

| Step | Data unit | What happens |
| --- | --- | --- |
| 1 | Data | Created at the Application layer. |
| 2 | Segment | Transport layer adds a TCP/UDP header. |
| 3 | Packet | Network layer adds an IP header. |
| 4 | Frame | Data Link layer adds MAC addresses. |
| 5 | Bits | Physical layer transmits signals. |

Summary: Encapsulation wraps data with headers at each layer as it goes down. Decapsulation reverses the process when receiving.

# 18. TCP vs UDP Comparison   [1:17:30]

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection | Connection-oriented | Connectionless |
| Speed | Slower (reliable) | Faster (no overhead) |
| Reliability | Guaranteed delivery | No guarantee |
| Error checking | Yes, extensive | Minimal |
| Ordering | Maintains packet order | No ordering |
| Use case | Web, Email, File Transfer | Gaming, Streaming, DNS |

Key differences:

- TCP Reliable, ordered, connection-based.

- UDP Fast, connectionless, best-effort.

Remember: TCP = Web / File Transfer ¦ UDP = Gaming / Streaming / DNS

# 19. TCP 3-Way Handshake (Connection Establishment)   [1:21:41]

Definition: Establishes a reliable TCP connection between two devices.

| Step | Direction | Message |
| --- | --- | --- |
| 1 | Client → SYN → Server | "I want to connect" (SYN packet sent). |
| 2 | Server → SYN-ACK → Client | "Acknowledged, you may connect" (server responds with SYN-ACK). |
| 3 | Client → ACK → Server | "Connected" (client sends the final ACK). |

Final status: "Connection Established ✓"

# 20. TCP 4-Way Handshake (Connection Termination)   [1:25:27]

Definition: Gracefully terminates a TCP connection between two devices.

| Step | Direction | Message |
| --- | --- | --- |
| 1 | Client → FIN → Server | "I'm done sending data" (FIN packet sent). |
| 2 | Server → ACK → Client | "Got it, I acknowledge" (server sends ACK). |
| 3 | Server → FIN → Client | "I'm done too" (server sends its own FIN). |
| 4 | Client → ACK → Server | "Confirmed, connection closed" (client sends the final ACK). |

Final status: "Connection Closed ✓"

# 21. Real-World Example: Opening Google.com   [1:27:38]

Flow: Browser (google.com)  →  DNS  →  TCP 3-way handshake  →  HTTP GET  →  google.com

| Stage | What happens |
| --- | --- |
| 1. DNS query sent | Browser requests the IP address for google.com. |
| 2. DNS response received | IP address is returned to the browser. |
| 3. TCP handshake | SYN, SYN-ACK and ACK packets establish the connection. |
| 4. HTTP request sent | GET request for the webpage content. |
| 5. Server processes request | Web server handles the request and retrieves the data. |

# 22. Common TCP Flags   [1:27:43]

TCP flags control the state and behavior of communication.

| Flag | Meaning |
| --- | --- |
| SYN | Initiates a new connection (synchronization). |
| ACK | Acknowledges receipt of data or packets. |
| FIN | Gracefully closes a connection. |
| RST | Forces an immediate connection reset. |
| PSH | Pushes data immediately without buffering. |
| URG | Marks data as urgent and high priority. |

Note: TCP flags are used in Nmap scans, port scanning, firewall evasion and TCP manipulation attacks.

# 23. Common Protocols Quick Reference   [1:29:39]

| Protocol | Port | Purpose |
| --- | --- | --- |
| HTTP | 80 | Unencrypted web traffic |
| HTTPS | 443 | Encrypted web traffic (SSL/TLS) |
| FTP | 21 | File transfer (unencrypted) |
| SFTP | 22 | Secure file transfer (SSH) |
| SSH | 22 | Secure shell, remote access |
| Telnet | 23 | Remote access (unencrypted, deprecated) |
| DNS | 53 | Domain name resolution |
| DHCP | 67-68 | Dynamic IP assignment |
| SMTP | 25 | Send email |
| POP3 | 110 | Receive email (older) |
| IMAP | 143 | Receive email (modern) |
| RDP | 3389 | Remote desktop (Windows) |

Important: Memorizing ports is essential for CTFs, pentesting and network security assessments.

# 24. Encryption & Decryption Process   [1:37:30]

Definition: Converting plaintext into ciphertext (encryption) and back (decryption) using cryptographic keys.

|  | Encryption | Decryption |
| --- | --- | --- |
| Step 1 | Plaintext (readable data) + encryption key. | Ciphertext (encrypted data) + decryption key. |
| Step 2 | Encryption algorithm (AES, RSA, DES). | Decryption algorithm (reverse of encryption). |
| Step 3 | Ciphertext (unreadable, scrambled data). | Plaintext (original readable data). |
| Result | Data is now secure and unreadable without the key. | Data is recovered and readable again. |

Instructor sketch: a sender types "Hi" (plain), it is encrypted into cipher text, travels across the network, and is decrypted at the receiver.

Key encryption types:

|  | Symmetric | Asymmetric |
| --- | --- | --- |
| Keys | Same key for encryption and decryption (AES, DES). | Different keys, public and private (RSA, ECC). |
| Speed / security | Fast and efficient. | Slower but more secure. |
| Key handling | Key must be shared securely. | Public key encrypts, private key decrypts. |
| Used for | Bulk data. | Key exchange and digital signatures. |

# 25. DNS: How Domain Resolution Works   [1:42:15]

Definition: Translates human-readable domain names into IP addresses.

- User types google.com in the browser.

- Query is sent to the Recursive Resolver (ISP DNS).

- Resolver queries the Root Nameserver.

- Root directs it to the TLD server (.com).

- TLD server directs it to the Authoritative Nameserver.

- Authoritative server returns the IP address (142.250.185.46).

- Browser connects to the IP and loads the website.

# 26. ARP: Address Resolution Protocol   [1:46:09 to 1:48:31]

Definition: Maps IP addresses to MAC addresses on a local network.

| Step | What happens |
| --- | --- |
| 1 | Device A wants to reach 192.168.1.5. It has the IP but needs the MAC address. |
| 2 | Broadcasts an ARP request: "Who has 192.168.1.5?" Sent to all devices on the local network (broadcast). |
| 3 | Device B replies: "I do. MAC: XX:XX:XX:XX:XX:XX". Device B sends an ARP reply with its MAC address. |
| 4 | Device A stores the result in its ARP cache (caches the IP-to-MAC mapping for future use). |
| 5 | Communication begins using the MAC address. Devices can now communicate on the local network. |

Instructor summary: ARP works inside a LAN and converts IP → MAC.

# 27. ICMP: Ping & Traceroute   [1:48:42]

Full form: Internet Control Message Protocol.

ICMP handles diagnostics, error reporting and network troubleshooting. Typical messages:

- Address not found.

- Packet expired in transit.

- Are you there? (Ping)

- Yes, I'm here! (ICMP echo reply)

Ping flow: Echo Request  →  Target  →  Echo Reply

- Measures Round-Trip Time (RTT) in milliseconds.

- Use Check if a host is reachable, measure latency.

- Example ping google.com → 25ms response.

# 28. HTTP Request & Response Cycle   [1:49:56]

Definition: HTTP is the protocol used to transfer data between browser and server.

- User enters a URL in the browser.

- Browser sends an HTTP request to the server.

- Server processes the request.

- Server sends back an HTTP response.

- Browser renders the page.

| HTTP request contains | HTTP response contains |
| --- | --- |
| Method (GET, POST, etc.) | Status code (200, 404, 500, etc.) |
| URL / Path | Headers (Set-Cookie, Content-Type) |
| Headers (Host, User-Agent, Cookie) | Body (HTML, JSON, etc.) |
| Body (optional, for POST/PUT) |  |

# 29. HTTP Methods & Headers   [1:52:17]

| Method | Purpose | Hacker relevance |
| --- | --- | --- |
| GET | Fetch data from server | Parameter tampering, URL manipulation |
| POST | Send data to server | SQLi, login brute force, form injection |
| PUT | Update existing resource | Unauthorized file upload, resource overwrite |
| DELETE | Remove resource | Unauthorized deletion, data destruction |
| PATCH | Partial update to resource | Privilege escalation, partial data modification |
| OPTIONS | Check allowed methods | Recon, discover available endpoints |

| Request headers | Response headers |
| --- | --- |
| Host | Set-Cookie |
| User-Agent | Server |
| Cookie | Content-Type |
| Authorization | X-Frame-Options |
| Content-Type | Access-Control-Allow-Origin (CORS) |
| Referer | Cache-Control |

# 30. HTTP Status Codes   [1:54:48]

Status codes tell you what happened with your request.

| Range | Meaning | Examples |
| --- | --- | --- |
| 1xx | Informational | 100 Continue, 101 Switching Protocols |
| 2xx | Success | 200 OK, 201 Created, 204 No Content |
| 3xx | Redirection | 301 Moved Permanently, 302 Found, 304 Not Modified |
| 4xx | Client error | 400 Bad Request, 401 Unauthorized, 403 Forbidden, 404 Not Found |
| 5xx | Server error | 500 Internal Server Error, 502 Bad Gateway, 503 Service Unavailable |

Key status codes breakdown:

| Code | Meaning |
| --- | --- |
| 200 OK | Request succeeded. |
| 201 Created | Resource created successfully. |
| 301 / 302 | Redirect to a new location. |
| 400 Bad Request | Malformed request. |
| 401 Unauthorized | Authentication required. |
| 403 Forbidden | Access denied (found but blocked). |
| 404 Not Found | Resource doesn't exist. |
| 500 Internal Server Error | Server misconfiguration or crash. |
| 503 Service Unavailable | Server overloaded. |

# 31. HTTPS & TLS Handshake   [1:56:30]

Definition: HTTPS = HTTP + TLS encryption layer for secure communication. (Instructor note: HTTP + SSL / TLS.)

TLS handshake steps:

- Client sends "Hello" with its supported TLS versions and ciphers.

- Server replies "Hello" with the chosen cipher and its SSL certificate.

- Client verifies the certificate (checks the CA trust chain).

- Client generates a session key and encrypts it with the server's public key.

- Server decrypts the session key with its private key.

- Both sides confirm, and the encrypted tunnel is established.

- HTTP data flows securely inside the encrypted tunnel.

| HTTP (unencrypted) | HTTPS (encrypted) |
| --- | --- |
| Plain text communication | TLS/SSL encryption |
| Easily interceptable | Requires a certificate |
| No authentication | Mutual authentication |
| Vulnerable to MITM attacks | Protected from MITM (if properly implemented) |

# 32. VPN: Virtual Private Network   [1:58:12]

Definition: A VPN creates an encrypted tunnel between your device and the internet.

- User connects to the VPN client.

- Traffic is encrypted on the device.

- Traffic is sent to the VPN server.

- VPN server decrypts it and forwards it to the internet.

- Response returns through the same encrypted tunnel.

# 33. Proxy Servers   [2:00:30]

Definition: A proxy sits between your device and the internet, forwarding requests on your behalf.

|  | Forward proxy | Reverse proxy | Transparent proxy |
| --- | --- | --- | --- |
| Side | Client side proxy | Server side proxy | Intercepts without the user knowing |
| Hides | Hides the user from the internet | Hides the server from the client | Transparent to the user |
| Used for | Anonymity and filtering | Load balancing and WAF | Corporate networks |
| Example | Corporate proxy | Cloudflare, Nginx | ISP monitoring |

Simple flow comparison:

- With proxy Device → Proxy → Internet

- Without proxy Device → Internet

# 34. Tor: The Onion Router   [2:02:53]

Definition: Tor anonymizes traffic by routing it through multiple encrypted relay nodes. (Instructor note: Tor Browser is used to reach the Dark Web, .onion sites.)

- User opens the Tor browser.

- Traffic is encrypted in 3 layers (like an onion).

- Sent to the Entry Node (knows your IP, not the destination).

- Passed to the Middle Node (knows nothing).

- Exit Node decrypts the final layer and connects to the destination.

- Destination sees the Exit Node IP, not your real IP.

| Tor characteristics | Security risks |
| --- | --- |
| Highly anonymous | Exit node traffic is unencrypted |
| Slow due to multiple hops | Exit node can be monitored |
| Decentralized network | Timing attacks possible |
| Free to use | Correlation attacks |